Smart Indoor Air Monitor with Fault Forecasting

Project Overview

## What the Project Does

This project is a smart system that watches the air inside a room. A small sensor device measures temperature, humidity and CO₂ levels and sends them to a computer. An AI model checks whether the air is behaving normally, explains what is most likely wrong, and predicts how risky the next 30 minutes will be. Everything is shown live on a web dashboard. Most monitors only report problems after they happen; this system aims to warn people early.

## How It Works

- Sense: An ESP32 sensor board takes a reading every 5 seconds and sends a 1-minute average over WiFi.

- Collect: A Python server gathers 5 minutes of readings and passes them to the AI model.

- Analyse: The model compares the latest readings with what “normal” looked like in past data, looking at levels, how jumpy they are, and whether they are rising or falling.

- Display: Results are pushed instantly to a live dashboard in the browser.

## Key Capabilities

- Anomaly detection: An Isolation Forest model, trained on real indoor sensor data, spots unusual air conditions and gives a probability that something is wrong.

- Simple alert levels: Each reading is marked Green (normal), Yellow (caution) or Red (critical).

- Cause identification: Estimates whether a problem is most likely due to heat, high CO₂, dust, or a faulty sensor.

- Risk forecasting: A Markov chain model predicts the chance of a fault 10, 15 and 30 minutes ahead, based on how problems have behaved in the past.

- Live dashboard: Shows current readings, trend charts, alert status, likely causes and future risk, updating in real time.

- Reliable by design: The sensor retries failed sends and reconnects to WiFi automatically; the dashboard runs in demo mode if the server is offline.

## Technology Used

- Hardware: ESP32-S3 microcontroller, DHT11 (temperature/humidity) and MQ135 (air quality) sensors

- Backend & AI: Python, FastAPI, scikit-learn, NumPy, pandas

- Frontend: React with live charts, connected via WebSockets

## Summary

The project combines low-cost hardware, machine learning and a real-time dashboard into one system that does more than measure air quality: it detects unusual conditions, explains them, and forecasts risk, so problems can be caught before they become serious.